YY-Y0109

Specifications: 
Sensor sensitivity distance / angle: 3m / 120 degree
Solar panel power: 0.48W, 17 percent efficiency
Solar panel size: 7.5 x 3.5cm ( L x W )
Built-in battery: 2.96WH
LED: 6 pieces SMD 2835 white light LED, 120Lux
LED power: 1W, 6000 - 6500K
Delay time: 10 seconds
Night sensor: Less than 10Lux
Charging time / Working time: 8 hours / more than 12 hours
Light mode: Off / High light
• High light: When people walk near the solar motion sensor light ( about 3m ), it will turn on High light mode

Operation Instructions: 
Unlock and activate the device
• The internal battery is locked for safety shipment, so users have to use the key pin to click the ON / OFF switch hole and unlock the battery and lighting system.
Install the device
• Please use the supplied screw sleeves and screws to mount the device on wall or somewhere else you want for lighting.
Charging via solar panel
• Please install the unit face south and make sure the solar panel can get the good sunlight directly without any shelter or glass. Normally it can be fully charged in one day under ideal bright sunlight.
Light control function
• It will auto turn on PIR sensor when in darkness or at night ( less than 10Lux ), and auto tum off when in daytime or bright area.

Special Note: 
• Keep the device away from fire.
• Please fully charge the device before first use.
• If there is no good sunlight in 3 months, the battery may be out of power, so you have to charge it by bright sunlight.
• If there is no good sunlight or in winter sunlight, especial in north corners, it may take longer time to fully charge the device and this is very normal for solar products.

Category: LED Outdoor Lights
Style: Modern, LED
Suggested Space Fit: Garden, Pathway, Outdoor
Number of Lights: 16 SMD 2835 LEDs
LED Color: White
Features: Sensor

Power Supply: Solar Powered
Color: white

Package weight: 0.227 kg
Product size (L x W x H): 12.5 x 9.5 x 4.8 cm / 4.91 x 3.73 x 1.89 inches
Package size (L x W x H) : 14 x 11 x 6 cm / 5.50 x 4.32 x 2.36 inches

Package Contents:

1 x YY601 Solar Light Motion Sensor 16 SMD 2835 LEDs IP65 Water Resistant Light Control Garden Lamp

2 x Screw

2 x Screw Sleeve

1 x Pin Key

1 x English User Manual

IPXX Rating Chart

IP Code
The IP Code (or International Protection Rating, sometimes also interpreted as Ingress Protection Rating*) consists of the letters IP followed by two digits and an optional letter. As defined in international standard IEC 60529, it classifies the degrees of protection provided against the intrusion of solid objects (including body parts like hands and fingers), dust, accidental contact, and water in electrical enclosures. The standard aims to provide users more detailed information than vague marketing terms such as waterproof.

The digits (characteristic numerals) indicate conformity with the conditions summarized in the tables below. For example, an electrical socket rated IP22 is protected against insertion of fingers and will not be damaged or become unsafe during a specified test in which it is exposed to vertically or nearly vertically dripping water. IP22 or 2X are typical minimum requirements for the design of electrical accessories for indoor use.

http://www.ebay.com/itm/6-LED-Solar-Powered-Motion-PIR-Light-Sensor-Outdoor-Garden-Wall-Lamp-Waterproof-/361353511409?hash=item54225901f1

http://www.amazon.co.jp/dp/B00RREP7K0