深圳市帕骐电子科技有限公司

产品部-10新品上架描述表

1.Key Features

（Red marked content must be show on Listing Title or Bullet Point ）

产品描述（字体颜色为红色代表要填写在标题或者短描述Bullet Point ）

1）Android 7.1 Nougat, CPU: Octa-Core,2GB RAM & 32GB ROM, High quality radio IC: NXP TEF6686, with good radio reception.

安卓7.1操作系统,八核CPU，内存2G+32G，高品质的6686收音IC，效果更好，

2）Built- in TDA7560 amplifier IC，Supports subwoofer audio output, and control it separately.

Supports 4K(3840*2160 pixels) /1080P HD video, supports phone mirroring function. You can use your smartphone's online GPS to mirror on this unit, support DAB+ function

支持重低音输出，并可以单独调节，支持播放4K/1080P 视频，支持手机互联, 可以把手机的在线导航显示在机器上，支持DAB数字收音机，

3）Supports GPS Navigation,pre-install Google map(without offline map data), Built in USB Port/ Micro SD Slot(up to 64GB), WIFI(Built in WIFI modem)/3G(Extra device required), Support reversing camera(Extra device required)

支持GPS功能，预装了谷歌地图（没有地图数据，需联网）,内置USB/SD卡槽（最大支持64GB），内置wifi模块，支持3G网络（要求额外的设备），支持倒车摄像头功能（摄像头选配）.

- 4) Please compare the dimensions, shape of our unit before purchasing, in case it does not fit for your car.

购买前还请先核对机子的形状和尺寸.

5) Support output almost everything to the headrests,even it play the video from USB/SD card,or online Youtube video.Supports Bluetooth call /audio play, Sync phonebook,quick search contacts, external microphone, comes with 1.5m long WIFI antenna for better signal reception.

支持几乎所有界面输出到头枕,支持蓝牙通话，蓝牙音乐，支持同步电话本,快速搜索联系人和支持外置麦克风,配有1.5米长的wifi天线，让机子更好的接收wifi信号.

- Technical Specification技术参数

Item Description:

7" 1024*600 Screen Octa-Core 7.1 Nougat Car Radio DVD Player for Benz A/B Series

Applicable Models：

The following car models and years are for reference, please make sure your original car radio has the same shape as our unit.

Benz A class W169 2004-2012

Benz W639 2006-2012

Benz B class W245 2005-2012

Mercedes-Benz Viano 2006-2012

Mercedes-Benz Vito 2006-2012

Benz W906 2006-2012

Note: If your car has Fiber Optic amplifier or other amplifier built-in,then your car has digital sound processor,this stereo DO NOT support that,you may need manually connect speakers cable directly to the car speakers to make it work,please ask technician to modify this if you do not know how to do!

Specifications：

General:

OS: Android 7.1 Nougat

CPU: Octa-Core

RAM: DDR3 2GB

Nand Memory: 32GB

Radio IC: NXP TEF6686

Amplifier IC: TDA7560

Bluetooth: BT 4.0

Working Voltage: 10.8~14.8V DC

Audio output: 4*45W Max

Cold Boot time: about 28s

Screen:

Screen Size: 7-Inch Digital Screen

Screen Resolution: 1024*600

Capacitive Touch Screen (5 points)

Multi-color button light

Panel buttons light RGB color are available, you can change it freely to match your car.

CPU Processor

Octa-core CPU processor with 2GB RAM&32GB ROM, which will provide you a more stable and faster user experience than before.

DAB+ Digital Radio (Extra DAB+ box Y0251 required)

Digital Audio Broadcasting allows you multi-regional music with your local well-known and also new channels. CD sound quality and noise-free in excellent transmission make DAB Radio a special experience in the vehicle.

Which make DAB broadly accessible to most of Europe.

AV Output

For this unit, it can output almost everything to your headrests, even you play the video from USB/SD /online youtube video, Many headunits in the market can only output the DVD video to the headrest.

EQ

Built-in TDA7560 amplifier IC，Supports subwoofer audio output, and control it separately.

Max audio output: 4 X 45W.

Phone Mirroring

Mirror your phone on the 7 inch screen.

For Android phone, you can mirror and control on the unit via USB cable connection ,
Air Play For iPhone, you can just mirror on the unit via wifi  connection, can not control

(Note: can support Android 7.0)

GPS：

Supports online and off-line navi app, supports 3D Maps and voice guidance, your long journey will no longer be bothered by the nightmare of getting lost.

Voice Guidance: Yes

3D Map: Yes

(Support radio/Bluetooth/USB/SD playback while navigating.)

Bluetooth 4.0:

Built-in Microphone in the control panel for hands free calling function

External microphone also included for more free clear phone call

Supports Bluetooth audio player/Phone Book/quick search contacts function

(Attention: This function can only be realized when the mobile phone supports Bluetooth Audio Output/Contacts Output)

Supports Bluetooth Connection for iPhone/ Most Android Phones/ Even some None Smart Phone

(Attention: The Bluetooth just for audio stream transfer, it doesn’t support file transfer or Internet tethering.)

Steering Wheel Control

Change the track, volume, etc. conveniently without taking your hands off the steering wheel, which makes your driving much safer.

Radio

High quality radio IC: NXP TEF6686, with good radio reception.

AM/FM tuner built in (worldwide)

AM frequency range: 522-1620 (Europe), 530-1710 (America), 522-1620 (Russia)

FM frequency range: 87.5-108 (Europe), 87.5-107.9 (America), 65.0-108.0 (Russia)

RDS Radio: Yes

USB/SD:

Built in micro SD card slot/ USB Port

Supports SD card/ USB drive up to 64GB with FAT32 or ntfs format

Supports Audio Formats: MP3/WMA/WAV/APE/FLAC/OGG/AC3, etc.

Supports Video Formats: RMVB/AVI/MPG/MPEG/VOB/MOV/ASF/FLV/3GP/MP4, etc.

OSD Language:

English, Russian, Spanish, Turkish, Portuguese, Arabic, Chinese, Italian, German, French, Thai, Hebrew, Danish，Hungarian，Dutch，Polish，Finnish，Greek，Bulgarian，Belgian，Swedish，etc.

OBD2 (Extra device required)

Compatible: Most of Bluetooth type ELM327 OBD2 scanner and wifi OBD2 (Search Y0004/Y0034 on our store)

Compatible Application: Torque

Display Data: Real-time data and trouble codes from vehicles computer.

(Attention: Few cars may not support the ECU reading accessing, those cars will not work this function.)

WIFI/3G internet

3G: Require external USB 3G modem (Search Y0014 on our store) and your local Internet SIM card to be connected，Only support WCDMA 3G bandwidth

WIFI: Built in WIFI modem

Direct turn on WIFI function to search and connect to any free WIFI hotspots

（It comes with 1.5m long WIFI antenna for better signal reception.）

Cam-In(Extra device required)

Supports reverse camera input, automatically trigger to reverse camera when reversing the car. (Only support RCA type reverse camera input.)

DVD

Support all formats

DVD drive: Region Free

Compatible Formats: AVI/DVD/DVD+R/DVD-R/DVD+RW/DVD-RW/VCD/MP3/CD/CD-RW/JPEG etc.

- Instruction 使用说明

- Package包装

5.Reference Link参考链接